Contents

Problem geometry	4

Enclosure:	4

Body of influence	4

Problem meshing	5

Problem setup	6

General	6

Model	6

Body force formulation	6

Surface tension	6

Surface tension force modelling	6

Turbulence model	6

Boundary conditions	6

Solution method	7

Solution controls:	7

Initialization:	7

Calculation	7

Results	7

The results were taken after 1.6 seconds	7

Validation	10

Validation with the paper (al Y. e., 2022)	10

Validation with the experiment (al, Experimental investigation of supercavitating flows, 2012)	10

Mesh convergence analysis	11

Simulation Data	11

Calculation of Pressure Differences	11

Results and Discussion	11

Mesh Convergence: Pressure Values	11

Mesh Convergence: Pressure Difference	12

Conclusion	13

Problem geometry	14

Enclosure:	14

Body of influence	14

Problem meshing	15

Problem setup	16

General	16

Model	16

Homogeneous Models	16

volume fraction parameters formulation	16

Body force formulation	16

Turbulence model	16

Boundary conditions	16

Solution method	17

Solution controls:	17

Initialization:	17

Calculation	17

Results	17

The results were taken after 1.6 seconds	17

Problem geometry	20

Enclosure:	20

Body of influence	20

Problem meshing (using Fluent mesher)	21

Problem setup	22

General	22

Model	22

Homogeneous Models	22

volume fraction parameters formulation	22

Body force formulation	22

Turbulence model	22

Boundary conditions	22

Solution method	23

Solution controls:	23

Initialization:	23

Calculation	23

Results	23

The results were taken after 1.6 seconds	23

Comparison between Cavitator Design 40 deg and 60 deg	26

40deg	26

60deg	26

References	27

# CFD simulation of ventilated cavitation

# Problem geometry

Figure 1

Figure 2

### Enclosure:

| X- | 158mm |
| --- | --- |
| X+ | 632mm |
| Y- | 75mm |
| Y+ | 75mm |
| Z- | 75mm |
| Z+ | 75mm |

### Body of influence

53.3*50*350 mm

# Problem meshing

Number of elements: 1683729

Body of influence is used to refine cell size around the cylinder with element size = 2mm.

30 inflation layers are added above cylinders walls with growth rate 1.05 and transition ratio 0.272.

Figure 3

Figure 4

Figure 5

# Problem setup

## General

Gravity=-9.81 m/s2

Energy: off

## Model

Multiphase VOF model with no mass transfer between the two phases

Primary phase: Water, Secondary phase: Air

Body force formulation

implicit body force

### Surface tension

constant coefficient = 0.072 N/m

### Surface tension force modelling

Model: continuum surface force

Adhesions options: wall adhesion

## Turbulence model

k-omega SST

## Boundary conditions

Equation: Cq=qvinf dc2

Where,

dc is the cavitator diameter

vinis the free stream velocity

q is the voumetric flow rate

| Inlet name | Type | Value | Air phase fraction |
| --- | --- | --- | --- |
| Main inlet | velocity | 6m/s | 0 |
| Inlet slot 1 | Mass flow | Expression | 1 |
| Inlet slot 1 | Mass flow | Expression | 1 |
| Inlet slot 1 | Mass flow | Expression | 1 |
| Inlet slot 1 | Mass flow | Expression | 1 |
| Outlet | Pressure outlet | 82 Kpa-gauge pressure | Backflow air VF 1 |
| Far-field | Velocity inlet in x | wall | 0 |

Expression

## Solution method

Couped algorithm with all spatial discretization settings as default.

### Solution controls:

All under-relaxation factors are left as default.

## Initialization:

Hybrid initialization.

## Calculation

Number of time step :16000, time step size :0.0001, Max iterations=20

# Results

## The results were taken after 1.6 seconds

Froude number Fn=V∞g*dc = 15.24011356

Cavitation number =σc=P-Pc 0.5ρv∞2

Figure 6 : Air VF iso surface 0.5

Figure 7 Air VF

Figure 8 Pressure distribution

Figure 9 Velocity contour

## Validation

### Validation with the paper (al Y. e., 2022)

### Validation with the experiment (al A. e., Experimental investigation of supercavitating flows, 2012)

# Mesh convergence analysis

The simulations were performed using different mesh sizes to observe the effect on the calculated cavity pressure. The mesh sizes used are:

- Mesh 1: 1,129,520 elements

- Mesh 2: 1,406,609 elements

- Mesh 3: 1,683,729 elements

- Mesh 4: 2,093,996 elements

### Simulation Data

The following cavitation numbers and cavity pressures were calculated for different mesh sizes at CQ=0.12 :

- Mesh 1: Cavitation Number = 0.439680, Pressure = 77.1095 kPa

- Mesh 2: Cavitation Number =0.400356, Pressure = 77.8152 kPa

- Mesh 3: Cavitation Number = 0.391753, Pressure = 77.9696 kPa

- Mesh4: Cavitation Number =0.388722, Pressure = 78.024kPa

### Calculation of Pressure Differences

Pressure differences were calculated to compare the simulation results with experimental and paper data:

CNU   PC1= 79 kPa
Paper PC1 =78.8kPa

## Results and Discussion

### Mesh Convergence: Pressure Values

The first plot shows the actual cavity pressure values for different mesh sizes:

Plot Description

- This plot helps to ensure that the pressure values stabilize and do not change significantly with finer meshes.

### Mesh Convergence: Pressure Difference

The second plot shows the pressure difference as a function of the number of mesh elements:

- Pressure differences between the simulation and paper data.

- Pressure differences between the simulation and experimental data.

- All simulations were run performed at a CQ value of 0.12

Plot Description

- This plot helps to determine the mesh size at which the pressure difference stabilizes, indicating mesh independence.

- A decrease in pressure difference with increasing mesh size suggests convergence.

## Conclusion

The mesh convergence analysis confirms that the CFD simulation results for super-ventilated cavitation in an underwater vehicle become mesh-independent as the mesh size increases. The pressure differences between the simulation and experimental data decrease with finer meshes, indicating convergence. The results demonstrate the importance of performing mesh convergence analysis to ensure accurate and reliable CFD simulations.

# CFD simulation of Cavitator Design 40 deg angle

# Problem geometry

Figure 1

Figure 2

### Enclosure:

| X- | 50cm |
| --- | --- |
| X+ | 349.4cm |
| Y- | 54cm |
| Y+ | 54cm |
| Z- | 54cm |
| Z+ | 54cm |

### Body of influence

30*30*225 cm

# Problem meshing

Number of elements: 4161474

Body of influence is used to refine cell size around the cylinder with element size = 9.5mm.

30 inflation layers are added above cylinders walls with growth rate 1.21 and first-layer thickness is 0.0047 mm.

Figure 3

Figure 4

Figure 5

# Problem setup

## General

Gravity= -9.81 m/s2 in y-direction

Energy: off

## Model

Multiphase VOF model with no mass transfer between the two phases

Primary phase: Water, Secondary phase: Air

Homogeneous Models

volume of fluid

volume fraction parameters formulation

implicit

Body force formulation

implicit body force

## Turbulence model

k-omega SST

## Boundary conditions

Equation: Cq=qvinf dc2

Were,

dc is the cavitator diameter

vinis the free stream velocity

q is the voumetric flow rate

| Inlet name | Type | Value | Air phase fraction |
| --- | --- | --- | --- |
| Main inlet | velocity | 9.17 m/s | 0 |
| Inlet vents | Mass flow | 30 m/s | 1 |
| Outlet | Pressure outlet | zero-gauge pressure | Backflow air VF 1 |
| cavitator-walls | wall |  |  |
| Far-field | wall |  |  |

## Solution method

Couped algorithm with all spatial discretization settings as default.

### Solution controls:

All under-relaxation factors are left as default.

## Initialization:

Hybrid initialization.

## Calculation

Number of time step :3200, time step size: 0.0005, Max iterations=20

# Results

## The results were taken after 1.6 seconds

Froude number Fn=V∞g*dc = 13.22625197

| Cavitation number calculation |  |  |
| --- | --- | --- |
| Pinf | 1.27866 | kpa |
| Pcavity | -1.26114 | kpa |
| vinf | 9.17 | m/s |
| rho (water) | 999 | kg/m^3 |
| Cavitation numebr | 0.060405044 |  |
| Sensor Location |  |  |
| x | y | z |
| 0.744 m | 0 | 0 |

Figure 6 : Air VF iso surface 0.5

Figure 7 Air VF

Figure 8 Pressure distribution

Figure 9 velocity distribution

Figure 10 drag with time

# CFD simulation of Cavitator Design 60 deg angle (Kim)

# Problem geometry

Figure 1

Figure 2

### Enclosure:

| X- | 50cm |
| --- | --- |
| X+ | 349.4cm |
| Y- | 54cm |
| Y+ | 54cm |
| Z- | 54cm |
| Z+ | 54cm |

### Body of influence

30*30*225 cm

# Problem meshing (using Fluent mesher)

Number of elements: 10184237

Body of influence is used to refine cell size around the cylinder with element size = 10mm.

30 inflation layers are added above cylinders walls with growth rate 1.21 and first-layer thickness is 0.0047 mm.

Figure 3

Figure 4

Figure 5

# Problem setup

## General

Gravity= -9.81 m/s2 in y-direction

Energy: off

## Model

Multiphase VOF model with no mass transfer between the two phases

Primary phase: Water, Secondary phase: Air

Homogeneous Models

volume of fluid

volume fraction parameters formulation

implicit

Body force formulation

implicit body force

## Turbulence model

k-omega SST

## Boundary conditions

Equation: Cq=qvinf dc2

Were,

dc is the cavitator diameter

vinis the free stream velocity

q is the voumetric flow rate

| Inlet name | Type | Value | Air phase fraction |
| --- | --- | --- | --- |
| Main inlet | velocity | 9.17 m/s | 0 |
| Inlet vents | Mass flow | 30 m/s | 1 |
| Outlet | Pressure outlet | zero-gauge pressure | Backflow air VF 1 |
| cavitator-walls | wall |  |  |
| Far-field | wall |  |  |

## Solution method

Couped algorithm with all spatial discretization settings as default.

### Solution controls:

All under-relaxation factors are left as default.

## Initialization:

Hybrid initialization.

## Calculation

Number of time step :3200, time step size: 0.0005, Max iterations=20

# Results

## The results were taken after 1.6 seconds

Froude number Fn=V∞g*dc = 10.48977331

| Cavitation number calculation |  |  |
| --- | --- | --- |
| Pinf | 1.69 | kpa |
| Pcavity | -2.26854 | kpa |
| vinf | 9.17 | m/s |
| rho (water) | 999 | kg/m^3 |
| Cavitation numebr | 0.094147485 |  |
| Sensor Location |  |  |
| x | y | z |
| 0.744 m | 0 | 0 |

Figure 6 : Air VF iso surface 0.5

Figure 7 Air VF

Figure 8 Pressure distribution

Figure 9 velocity distribution

Figure 10 drag with time

| # Comparison between Cavitator Design 40 deg and 60 deg |  |  |
| --- | --- | --- |
| angle | ## 40deg | ## 60deg |
| Cavitation numebr | 0.060405044 | 0.094147485 |
| Drag | At time t=0.2 sec Drag=19.9186 N | At time t=0.655 sec Drag=62.5733 N |
| Cavity width | Width=0.0945688 m | Width=0.1407732 m |

# References

al, A. e. (2012). Experimental investigation of supercavitating flows.

al, A. e. (n.d.). Experimental investigation of ventilated supercavitation behind cone-shaped with different angles and disk-shaped cavitators.

al, Y. e. (2022). Computational investigation on ventilated supercavitating flows and its.

Kim, M.-J. (n.d.). Cavitator Design for Straight-Running Supercavitating.